Annex A

مواصفات الأثاث المكتبي، مركز معالجة الأورام

Office Furniture, Oncology Centre

N.B. The following are the minimum requirements for all office furniture items. Offers specifying higher quality engineered wood and materials will be considered. All pictures shown are only meant as illustrations for general design and shape. All dimensions herein specified are approximate.

Item #: A-1: Office Desk-W160						QTY: 65

- Specifications:

  - 4-legged desk - square steel frame(50 x 50 mm).

  - Tops shall be made of 30 mm melamine-faced(on both sides)chipboard, tested forwater, heat, and scratch resistance.

  - Edge: P.V.C, thickness: 2 mm.

  - Paint of frame: Powder coat.

  - Smart invisible horizontal & vertical cable management (cable channel, cable outlet).

- Dimensions:

  - Width: 160 cm

  - Depth:    70 cm

  - Height:  75 cm

- Finishes: Highest quality selection of stains and colors from standard manufacturer’s range. All availableconfigurations must be provided in proposal.

Item #: A-2: Office Desk-W180					QTY: 20

- Specifications:

  - 4-legged desk - square steel frame (50 x 50 mm).

  - Tops shall be made of 30 mm melamine-faced (on both sides)chipboard, tested for water, heat, and scratch resistance.

  - Edge: P.V.C, thickness: 2 mm.

  - Paint of frame: Powder coat.

  - Smart invisible horizontal & vertical cable management (cable channel, cable outlet).

- Dimensions:

  - Width: 180 cm

  - Depth:    70 cm

  - Height:  75 cm

- Finishes: Highest quality selection of stains and colors from standard manufacturer’s range. All availableconfigurations must be provided in proposal.

Item #: A-3: Side table for Office Desk				QTY: 85

- Specifications:

  - 4-legged table - square steel frame (50 x 50 mm).

  - Tops shall be made of 30 mm melamine-faced (on both sides)chipboard, tested for water, heat, and scratch resistance.

  - Edge: P.V.C, thickness: 2 mm.

  - Paint of frame: Powder coat.

  - Smart invisible horizontal & vertical cable management (cable channel, cable outlet).

  - Possibility to choose various colors for tops and legs to match design theme.

- Dimensions:

  - Width: 100 cm

  - Depth:   70 cm

  - Height:  75 cm

- Finishes: Highest quality selection of stains and colors from standard manufacturer’s range. All availableconfigurations must be provided in proposal.

Item #: A-4:Drawer Unit					QTY: 85

- Specifications:

  - 3-drawer mobile pedestal mounted on 4 high quality castors.

  - Central lock.

  - Unit shall be made of 18and top of 30 mm melamine-faced(on both sides)chipboard, tested for water, heat, and scratch resistance.

  - Fronts shall be made of 18 mm melamine-faced (on both sides)laminate board.

- Dimensions:40 x 47 x 63 cm.

- Finishes: Highest quality selection of stains and colors from standard manufacturer’s range. All availableconfigurations must be provided in proposal.

Item #: A-5: End Table						QTY: 85

- Specifications:

  - 4legs connected by a frame, square steel (50 x 50 mm).

  - Wooden table top made from 18 mm thick melamine-faced (on both sides) chipboard, tested for water, heat, and scratch resistance.

  - Edge band: P.V.C, thickness: 2 mm.

  - Paint of frame: Powder coat.

- Dimensions:55 x 55 x 42 cm

- Finishes: Highest quality selection of stains and colors from standard manufacturer’s range. All availableconfigurations must be provided in proposal.

Item #: A-6: Middle Table					QTY: 50

- Specifications:

  - 4legs connected by a frame, square steel (50 x 50 mm).

  - Wooden table top made from 18 mm thick melamine-faced (on both sides) chipboard, tested for water, heat, and scratch resistance.

  - Edge band: P.V.C, thickness: 2 mm.

  - Paint of frame: Powder coat.

- Dimensions:80 x 55 x 42 cm

- Finishes: Highest quality selection of stains and colors from standard manufacturer’s range. All availableconfigurations must be provided in proposal.

Item #: A-7: CHAIR	, EMPLOYEE				QTY: 150

- Specifications:

  - Wide seat and backrest, soft upholstered seat with injected foam.

  - Seat width: 47cm

  - Seat depth: 46cm

  - Gas action to adjust the height

  - Frame: plastic base on wheels

  - Durable armrests

  - Capability to support user weight up to 150 KG.

- Finishes: Highest quality selection ofupholstery and frametypes and colors from standard manufacturer’s range. All available configurations must be provided in proposal.

Item #: A-8: Chair, Visitor						QTY: 200

- Specifications:

  - Wide seat and backrest, soft upholstered seat with injected foam.

  - Seat width: 47cm

  - Seat depth: 46cm

  - Plastic shield under the seat

  - Frame: Metal cantilever base with chrome or powder coat finish

  - Durable armrests

  - Capability to support user weight up to 150 KG.

- Finishes: Highest quality selection ofupholstery and frametypes and colors from standard manufacturer’s range. All available configurations must be provided in proposal.

Item #: A-9:Lab Stool						QTY: 60

- Specifications:

  - Seat width: 46cm

  - Seat depth:44-46cm

  - Seat and backrest: Non-slippery seat and backrest surface.

  - Smoothly adjustable chair height by means of a gas lift or screw mechanism.

  - Base: Steel with nylon covers, steel ring foot rest with powder coat or chrome finish.

- Finishes: Highest quality selection of frame types, materials and colors from standard manufacturer’s range. All available configurations must be provided in proposal.

Item #: A-10: Armchair					QTY: 75

- Specifications:

  - Smooth splash-proof upholstered single-seat lounger.

  - Seating dimensions (approximate): 56 × 55 cm

  - Height (approximate): 80cm, seat height: 45cm

  - Durable wooden legs.

  - Capability to support weight of up to 150 KG.

- Finishes: Highest quality selection ofupholstery and frametypes and colors from standard manufacturer’s range. All available configurations must be provided in proposal.

Item #: A-11: Double Sofa					QTY: 60

- Specifications:

  - Smooth splash-proof upholstered double-seat lounger.

  - Height (approximate): 80cm, seat height: 45cm

  - Durable wooden legs.

  - Capability to support weight of up to 200 Kg per person.

- Finishes: Highest quality selection ofupholstery and frametypes and colors from standard manufacturer’s range. All available configurations must be provided in proposal.

Item #: A-12: 3-Seater Sofa					QTY: 30

- Specifications:

  - Smooth splash-proof upholstered triple-seat lounger.

  - Height (approximate): 80cm, seat height: 45cm

  - Durable wooden legs.

  - Capability to support weight of up to 275 Kg per person.

- Finishes: Highest quality selection ofupholstery and frametypes and colors from standard manufacturer’s range. All available configurations must be provided in proposal.

Item#: A-13: Cafeteria chair					QTY: 160

- Specifications:

  - Seat and backrest made of Tecnoprene, i.e. polypropylene reinforced with fiberglass.

  - Must be Available in at least four colors.

  - 4- Legged chrome-plated metal frame.

  - Easy Stacking for up to 6 pieces.

  - Light enough to be easy to clean, store, and move around.

  - No sharp edges.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range.

Item #: A-14: 2-Seater Bench					QTY: 100

- Specifications:

- 2-Seater bench with connected frame resting on four legs.

- Padded seat and back.

- Frame material:steel with chrome or powder coat finish,or aluminum.

- Armrests with plastic pads.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range.

- Fire Retardant: Constructions to comply with Civil Defense/Local regulations regarding fire hazard.

Item#: A-15: 3-Seater Bench					QTY: 70

- Specifications:

- 2-Seater bench with connected frame resting on four legs.

- Padded seat and back.

- Frame material:steel with chrome or powder coat finish,or aluminum.

- Armrests with plastic pads.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range.

- Fire Retardant: Constructions to comply with Civil Defense/Local regulations regarding fire hazard.

Item#: A-16: Multipurpose Table					QTY: 50

- Specifications:

- Occasional Tables of laminate melamine-faced (on both sides)board (30mm).

- High quality durable table with a strong and stable structure

- Frame material:steel with chrome or powder coat finish,or aluminum.

- Various shape, sizes and heights.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range.

Item#: A-17: Metal Shelves					QTY: 75

- Specifications:

  - Slotted angles metal shelving unit.

  - Approx. External dimensions. (H 250 x W 91 x D 31) cm.

  - (5) Metal shelves.

  - Shelf load capacity (40 Kg) UDL.

  - Edges are flanged to increase rigidity,

  - (4) Rubber shoes for angles, Bolts and nuts for shelves.

  - Electro static powder coat finish, color gray

  - Easy to build any configuration and organize any workspace.

  - Easy to assemble and disassemble.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range.

- Fire Retardant: Constructions to comply with Civil Defense/Local regulations regarding fire hazard.

Item#: A-18: Long Span Shelving Unit (Starter Bay)			QTY: 100

- Specifications:

  - Approxime. Externaldimensions (H 250xW112xD 64) cm.

  - An upright width of 60mm with a central tapered slot that offers strong friction connection.

  - Easy to assemble and disassemble.

  - The heavy-duty beams are roll formed as a step down `Z` profile section with a four-ﬁngered claw that can be adjusted on a pitch of 40 mm.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range.

- Fire Retardant: Constructions to comply with Civil Defense/Local regulations regarding fire hazard.

Item#: A-19: Long Span Shelving Unit (Following Bay)			QTY: 300

- Specifications:

  - Approxime. External dimensions (H 250 xW112 x D 64) cm.

  - An upright width of 60mm with a central tapered slot that offers strong friction connection.

  - Easy to assemble and disassemble.

  - The heavy-duty beams are roll formed as a step down `Z` profile section with a four-ﬁngered claw that can be adjusted on a pitch of 40 mm.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range.

- Fire Retardant: Constructions to comply with Civil Defense/Local regulations regarding fire hazard.

Item#: A-20: FilingCupboard					QTY: 60

- Specifications:

  - Contemporary aesthetic filing storage system to match a range of themes.

  - Approximate external dimensions (H180xW 90xD 42) cm.

  - Flush handle witha 3-Point Lock.

  - Internalhinges.

  - Four adjustable shelves.

  - Powder coatfinish.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range.

- Fire Retardant: Constructions to comply with Civil Defense/Local regulations regarding fire hazard.

Item: #: A-21: 2-Tier LockerCabinet (1 Unit wide)		QTY: 350

- Specifications:

  - Durable2-tier 1-wide locker cabinet for employees.

  - Sheet metal construction, thickness ≥ 0.8mm.

  - Powder coat finish.

  - Approximate external dimensions (H185 xW 30 xD 45) cm.

  - 4 sturdy legs, length ≥ 10cm.

  - Internal Hinges.

  - At least one adjustable shelf.

  - Adequate number of vents.

  - Cylinder type key lock.

  - Provision for name/number plates.

  - 4 compatible engravable name/number plates with each unit.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item#: A-22: Conference Table					QTY: 5

- Specifications:

  - Approximate external dimensions (L 250 xW 160 x H75) cm.

  - Wooden Table Top: 18 mm thickness melamine-faced (on both sides) chipboardtested for water, heat, and scratch resistance.

  - PVC Edge Band:1mm on all the visible edges.

  - Metal Sheets, Connectors:Durable Metal legs with thick metal sheets on top to give sturdy and stable support to table top.

  - Paint: powder coat for all metal parts.

  - Easy to assemble and disassemble.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item#: A-23: Hinged Door Cabinet					QTY: 85

- Specifications:

  - Approximate external dimensions (W 80x D 40 x H85) cm.

  - At least one shelf.

  - Body:melamine-faced (onboth sides)chipboardtested for water, heat, and scratch resistance.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item#: A-24: FilingCabinet					QTY: 125

- Specifications:

  - 2-tiear cabinet with doors.

  - Approximate external dimensions (W 80 x D 40 x H192) cm.

  - Body: melamine-faced (onboth sides) chipboard tested for water, heat, and scratch resistance.

  - Equipped with key lock and two handles.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item#: A-25: Armchair Bed						QTY:150

- Specifications:

  - Single-seat comfortable lounger that transforms into a bed when needed.

  - Approximate dimensions (80 x 90 x 90) cm.

  - Armchair with its frame resting

  - Durable frame and conical spring-loaded foam cushions.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item#: A-26: Nurse Bed& Mattress				QTY: 30

- Specifications:

  - Metal Bed size 190x100cm.

  - Metal tube 2.5x5cm.

  - Middle support 13 tubes (2x2) cm.

  - 1mmpowder coat paint.

  - High-densitymattress:(L190x W90x H16) cm.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item#: A-27: Multipurpose chair					QTY: 130

- Specifications:

  - Seat and backrest: Soft upholstered.

  - Frame: Chrome-plated metal frame.

  - Suitable for a multitude of settings.

  - Durable rubber leg pads.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item#: A-28: Meeting Chair						QTY: 40

- Specifications:

  - Seat and backrest: soft upholstered seat with injected foam.

  - Approximate dimensions: W 47 x D 47 cm.

  - Gas action to adjust the height.

  - Frame aluminum   base on wheels.

  - Armrests:chrome.

  - Capability to support user weight up to 150 Kg.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item #: A-29:VIP 3-Seater Sofa					QTY: 2

- Specifications:

  - VIP 3-seater lounger for any waiting environment.

  - Chrome-plated legs.

  - Cushions with conical springs for maximum comfort.

  - Leather upholstery.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item #: A-30: VIP 2-SeaterSofa:					QTY: 4

- Specifications:

  - VIP 2-seater lounger for any waiting environment.

  - Chrome-plated legs.

  - Cushions with conical springs for maximum comfort.

  - Leather upholstery.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

Item #: A-31: VIPArmchair						QTY: 2

- Specifications:

  - VIP single-seat lounger for any waiting environment.

  - Chrome-plated legs.

  - Cushions with conical springs for maximum comfort.

  - Leather upholstery.

- Finishes: Highest quality selection oftypes and colors from standard manufacturer’s range. All options must be listed and priced in proposal.

الشروط الخاصة:

- ان تكون جميع المواد الموردة جديدة وصالحة 100% وخالية من أي عيوب في الصنع أو في المادة ومن طراز حديث لم توقف إنتاجها (القديم يرفض).

- بلد المنشأ:امريكيا،كندا،اليابان،بريطانيا،فنلندا، الدنمارك، سويسرا،بلجيكيا،المانيا،فرنسا،هولندا، تركيا، اليابان. ويستثنى من هذا الشرط الموادذوات الأرقام:

(Item #: A-17, Item #: A-18, Item #: A-19, Item #: A-21, Item #: A-26)

- تقديم مخططات shop drawing لتوزيع الاثاث Room by room furniture

- على المتقدمين للعطاء تقديم استعلاماتهم خلال الثلث الاول من فترة اغلاق العطاء.

- تقييم المناقص حسب الكفاءة والملاءة المالية.

- ان تحتوي العروض على كفالة مصنعية لمدة عاميين (سنتين).

- تقديم الكتيبات او أقراص (CD/DVD) يحتوي على (Operation Manual)و (servicemanual) عند التسليم.

- الاسعار شاملة الشحن، النقل الى الموقع، التركيب ومايحتاج من تأسيسات لضمان التركيب والتشغيل على أكمل وجه.

- مكان التسليم والتركيب والتشغيل في مركز معالجة الاورام /عمان وحسب تعليمات لجان الاستلام وشعبة الاثاث واللوازم الغير طبية.

- يتم توزيع الاثاث على المواقع حسب تعليمات لجان الاستلام وشعبة الاثاث واللوازم الغير طبيةوحسب المخططات لكل موقع.

- بيان أقرب مدة للتسليم.

- تقديم جميع البروشورات الفنية.

- تقديم شهادة منشأ للمواد.

- تقديم شهادات الجودة المصدقة اللازمة للمواد عند التسليم.

- تقديم صورة مصدقة عن شهادة المنشأ عند التسليم.

- تقديم كفالة صيانة بنكية بقيمة 5% من إجمالي قيمة الإحالة للمواد شاملة قطع الغيار العاملة صالحة لمدة سنتين من تاريخ التركيب والتشغيل.

- تقديم تعهد بتوفير قطع الغيار لمدة 5 سنوات من تاريخ التركيب والتشغيل للمواد.

- تقديم قائمة مسعرة بالقطع الاحتياطية صالحة لمدة خمس سنوات بعد انتهاء كفالة الصيانة للمواد.

- تقديم (compliance sheet) تحتوي على خانة تشير الى البروشور ويحق للجنة الشراء في حال عدم المطابقة، الرفض الجزئي او الكلي، علماً بأن المناقصين مسؤولون عن صحة (compliance sheet)، الاخطاء والمعلومات التضليلية تقود الشركة الى وضعها على القائمة السوداء مستقبلًا.

Annex B

الاثاث غير الطبي لمشروع مركز معالجة الاورام	مواصفات البرادي والستائر

لمشروع مركز معالجة الاورام

مواصفات البرادي والستائرلمشروع مركز معالجة الأورام

- تركيب ستائر في مختلف الطوابق التي يوجد فيها غرف المرضى، غرف عزل للمرضى، العناية الحثيثة،الخداج،الانعاش، المكاتب الادارية،العيادات، قاعات الانتظار، والاستراحات.

- ان تكون جميع الستائر، والجسور ذات نوعية وجودة عالية وبمعايير عالمية.

- ان تكون المواد المعدنية المستعملة في الستائر غير قابلة للصدأ وآمنة للاستعمال.

الانواع المطلوبة

Item B1: ستائر يدوية (operated by stainless steel chain system) نوع Roller وحسب المواصفات التالية:

- سهولة الاستخدام.

- سهولة التنظيف (ولا تتأثر سلبًا بمواد التنظيف).

- لا تأخذ حيز عند التركيب.

- تحمي من الحرارة الناتجة عن اشعة الشمس.

- ان تكون مقاومة للبكتيريا والعفن (Bioactive).

- ان تكون مقاومة للحريق (Flameretardant).

- غير قابلة لامتصاص المياه (water proof).

- ان تكون سهلة الصيانة.

- ان تسمح بمرور كمية مناسبة من الضوء وغير كاشفة.

- ان تكون الالوان متناسبة مع طبيعة الاقسام.

- لها قدرة عالية على تحمل الاستخدام المتكرر.

- ان يتناسب عرض الستائر مع المقطع الزجاجي للشبابيك.

: Item B2-1 ستائر مع جسر لأسرةالمرضى وحسب المواصفات التالية:

- سهولة الاستخدام والصيانة.

- سهولة التنظيف (ولا تتأثر سلبًا بمواد التنظيف).

- ان تكون مقاومة للبكتيريا والعفن (Bioactive).

- ان تكون مقاومة للحريق (Flame retardant).

- غير قابلة لامتصاص المياه (water proof).

- الجسر مصنوع من الالمنيوم محيطبالسرير مع مجرى لتثبيت الحلقات الحاملة للبرداية مع غطاء في اخر الجسر لغلقه، مع وجود cylindrical pole للتثبيت في السقف.

- ان تكون الحلقات المثبتة للبرداية من مادة معدنية ذات متانة عالية مع ضمان توفرها في السوق المحلي وتقديم تعهد بتوفرها في السوق المحلي لمدة 5 سنوات مع تثبيت سعرها.

- تقديم عينة مع الألوان المتوافرة وبالكميات المطلوبة وللجنة الشراء الحق في رفض اي عينة لا تراها مناسبة.

- السعر شامل التوريد والتركيب في الموقع مركز معالجة الاورام /عمان مع كل مايلزم لإتمام التركيب على أكملوجه.

- تقديم كفالة صيانة لمدة عامين من تاريخ الاستلام.

- على جميع الشركات المناقصة زيارة الموقعودراسة كافة المواقع التي تحتاج لتركيب ستائر للأسرة والقياسات المطلوبة وتقديم العروض المناسبة.

- أن يتضمن العرض تفاصيل كاملة لجميع المواقع التي تحتاج لتركيب ستائر للأسرة في كل طابق.

- تقديم عينة لكل نوع من الستائر المناقص عليها لأغراض الفحص المخبري في المختبر الصناعياو اي مختبر معتمد لدى القوات المسلحة الاردنية وعلى نفقة الشركة المناقصة وحسب التعليمات.

الشروط الخاصة:

- السعر شامل التوريد والتركيب في الموقع مركز معالجة الاورام /عمان مع كل ما يلزم لإتمام التركيب على أكمل وجه.

- بيان أقرب مدة للتسليم، وسيؤخذ ذلك بعين الاعتبار.

- ان يتضمن العرض كفالة صيانة مجانية لمدة عامين من تاريخ الاستلام.

- على جميع الشركات المناقصة زيارة الموقع ودراسة جميع المساحات والقياسات المطلوبة للبرادي وتقديم العروض المناسبة المفصلة للأنواع والقياسات والمساحات مع بيان النوع المقدم حسب طبيعة كل موقع.

- على جميع الشركات المناقصة استشارة شعبة الاثاث واللوازم غير الطبية في مديرية الخدمات الطبية الملكية لمزيد من التفاصيل والاستفسار يرجى الاتصال على هاتف رقم 5804804 فرعي 64435 أو دائرة التخطيط والدراسات في مديرية الخدمات الطبية الملكية.

- ان يتضمن العرض تفاصيل كاملة لجميع الطوابق وحسب القياسات لكل طابق.

- تقديم عينة لكل نوع من الستائر المناقص عليها وللجنة الحق في رفض اي عينة لا تراها مناسبة.

- تقديم عينة لكل نوع من الستائر المناقص عليها لأغراض الفحص المخبري في المختبر الصناعي او اي مختبر معتمد لدى القوات المسلحة الاردنية وعلى نفقة الشركة المناقصة وحسب التعليمات.

- ان يتضمن التركيب جميع البرادي والجسور والإكسسوارات ولجميع انواع الستائر المناقص عليها.

- ان يتضمن العرض تقديم كتالوجات للستائر المناقص عليها.

- بيان بلد المنشأ.

- تقديم كفالة عدلية لضمان سوء المصنعية.

- ان يتم وضع التصميم للستائر من قبل اشخاص مؤهلين ذوي خبرة في الديكور والتصميم الداخلي وأن تلائم التصاميم طبيعة كل موقع.

- تقديم شهادة جودة من الشركة الصانعة بان البرادي مقاومة للبكتيريا والعفن ولرطوبة والحريق.

Annex C

المواصفات الفنية والشروط الخاصة لاجهزة الحاسوب والطابعات ولوازم الكمبيوتر لمركز المعالجة بالاشعة في مستشفى الملكة علياء العسكري

- Note: These are minimum specifications but higher specifications are accepted for any  item below

Item(1): server

Item (1.1): server solution specification		_         QTY. (  1  )

Solution Should be scale-out unlimited numbers no restriction in IOPS

Minimum three (for N+1 redundancy) )Nodes configured with total Minimum resources as following: Cores = 48 Core RAM = 768 GB 1 x 1G SFP port per node for out of band management 4 x 10G SFP+ ports per node for production traffic Three years on- site warranty ( Hardware , software) ================================= For Hybrid configuration: - SSD Storage = 2 TB usable (before compression and reduplication) - HDD Storage = 21.4 TB usable (before compression and reduplication) For all flash configuration: - SSD 23.4 TB usable (before compression and reduplication)

Storage across all appliance nodes must be clustered and presented as a single, distributed file system to all the hypervisor hosts

The single file system must support both iSCSI and NFS protocols

All components of the hperconverge infrastructure should be licensed for the life time of the product ( not subscription)

Seamless Scale-Out Architecture with no downtime. No single Point of failure

Support Active/Active Data Centers

Per-VM snapshots and replication.

The solution must provide performance monitoring capabilities (real-time and historical)

The solution must be certified to run on standard x86 hardware.

The solution be scalable to accommodate for future expansion, minimum 16 nodes per cluster.

Must support iSCSI, NFS protocols

The solution must support both homogeneous and heterogeneous node configurations for CPU/RAM/HDD/SSD within the same cluster

The solution must be either a Hybrid solution with real-time storage auto-tiering, or an all-flash configurations or a combination of both.

Integrated data protection capabilities for backup, archiving, high-availability, and disaster recovery purposes

Support for both Block and File storage

The solution must be able to support: Hot and cold data Compression Hot and cold data Deduplication The solution must support both storage thin and thick provisioning

The solution must support point in time data protection capabilities for high availability and disaster recovery. The solution must be capable of Sync and Async replication. Ability to restore files from snapshots directly without starting the hypervisor.

The solution must support the capability that allows for external bare-metal workloads to access the hyper-converged storage.

Integrated enterprise-grade management for storage, compute, virtualization and data protection.

Support for single cluster and multi-cluster management

Integration with Microsoft Active Directory

Support for cluster Upgrades with no downtime.

similar references in Jordan

Local spares in Jordan with prove

All needed cables are included

The bidder must have at least two technical certified engineer on the provided system support

=================================================================================

Item(1.2) licenses

- All needed licenses for the above item(1.1) include hardware , software and hypervisor

- All needed licenses to run our oracle applications (Forms and reports) with Linux operating system on only one node of the server in item(1.1)

Item(2) : Personnel Computers

Item (2.1): Core i5 Desktop  PC’s 		_  		 QTY. (  150    )

| PROCESSOR (CPU) | Intel core i5-4590 (3.3 GHz , 6 MB, ) |
| --- | --- |
| CHIPSET | Intel Q87 |
| MEMORY ( RAM) | 4 GB DDR3-1600 MHz Up to 15 GB |
| HARD DISK | 500GB 7200 rpm SATA hard drive |
| DVD-RW DRIVE | Super Multi DVD writer |
| GRAPHIC | Integrated graphics-Intel® HD graphics |
| MONITOR | 19.5” LED Monitor |
| AUDIO | Integrated High Definition Audio , internal Speakers On Case or Monitor |
| I/O INTERFACE & PORTS | 4x USB 2.0, 4 xUSB 3.0 Ports, 1 x Ethernet (RJ-45), 1 x Audio in,1 x Audio out, 1 x VGA. |
| SLOTS | 1 PCLe (×16 graphics),1 PCLe(×1) |
| COMMUNICATION | Integrated 10/100/1000M Gigabit Ethernet |
| KEYBORD | Original Arabic /English keyboard |
| MOUSE | Original Optical mouse +mouse pad |
| POWER SUPPLY | 200—240 V AC ,50—60 Hz (Direct Power) |
| OPERATING SYSTEM | Free Dos |
| CASE | Tower Case (Micro or Mini) |
| WARRANTY | Three years warranty including Labor & spare parts |
| OTHERS | All drivers must be included . Monitor , Keyboard , Mouse and Case from the same Brand Only the Approved Brands from JAF . Your company must install windows using our JAF License |

Item (2.2): Core i7 Desktop  PC’s 		  __	  		 QTY. (  10    )

| Processor | Intel® Core™ i7-6700 with Intel HD Graphics (3.40 GHz ,8MB Cache, 4 cores) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Chipset | Intel Q170 |  |  |  |  |  |
| Form Factor | Tower |  |  |  |  |  |
| Memory | 8GB Up to 32GB DDR3L SDRAM, 1600 MHz, (4 DIMMS) |  |  |  |  |  |
| Hard Drive | 500 GB Up To 1 TB SATA SSHD |  |  |  |  |  |
| Optical Drive | SATA Super Multi DVD |  |  |  |  |  |
| Display | 21" LED-Monitor (Same Brand) |  |  |  |  |  |
| Graphics | Integrated Intel HD |  |  |  |  |  |
| Key Board | A/L USB Key board (Same Brand) |  |  |  |  |  |
| Mouse | USB Optical Mouse (Same Brand) |  |  |  |  |  |
| Audio | Integrated High Definition Audio |  |  |  |  |  |
| Communication | Ethernet: Integrated (10/100/1000) |  |  |  |  |  |
| Expansion Slot | 1 full-height PCIe x1, 1 full-height PCIe x16 |  |  |  |  |  |
| Ports | Input | Output | USB | VGA | Ethernet | Display |  |
|  | 1 Audio in 1 microphone | 1 audio out 1 Headphone | 3 X 3.0 2 X 2.0 | 1 VGA | 1 Rj-45 | 1 Display Port |  |
| Power supply | 100 ~ 240 VAC , 50-60 Hz. |  |  |  |  |  |
| Others | All Drivers And MK Power Cord That Support The System Must Be Provided |  |  |  |  |  |
| Warranty | 3-Year Warranty Including Spare Parts And Labor |  |  |  |  |  |
| OS | Free Dos |  |  |  |  |  |

Item (2.3): Operating system& software	     	       	       	QTY( 160  )

1. Microsoft Windows 7 professional license key for the above items (2.1) and (2.2) .

2. Price for OS & Software should be listed separately.

===================================================================

Item(3) : laptops

Item (3.1 )   :  Laptop specifications                                                        QTY(5)

|  | Feature | Minimum Specifications |
| --- | --- | --- |
| 1 | Processor | Intel® Core™ i7-6820HQ (2.7 GHz UP To 3.6GHz , 8MB Cache) . |
| 2 | Chipset | Intel integrated with processor |
| 3 | Memory | 8GB Up to 16GB , 1600 MHz DDR3 |
| 4 | Hard Drive | 500 GB SATA |
| 5 | Optical Drive | DVD +/-RW |
| 6 | Display | 15.6” HD (1366X768)Anti-Glare LED – Backlit Diagonal |
| 7 | Graphics | Integrated Intel HD |
| 8 | Ports | 2 USB 3.0,2 USB 2.0,1VGA,1 Stereo Microphone in, 1 Stereo Microphone /Line-out,1 AC Power,HDMI |
| 9 | Audio | High Definition Audio |
| 10 | Communication | Ethernet (10/100/1000) GIG NIC, Wireless 802.11 b/g/n |
| 11 | Battery | 3-cell(≥40 WHr) |
| 12 | Key Board | A/L Key board |
| 13 | Carrying Case | Same Brand |
| 14 | Others | All Drivers that support the system must be provided |
| 15 | Warranty | 3-Year Warranty Including Spare Parts And Labor |
| 16 | OS | Free Dos |

Item (3.2): Operating system& software		           		QTY( 5  )

1. Microsoft Windows 10  professional license key for the above item(3.1) .

2.Price for OS & Software should be listed separately.

==============================================================

Item(4) : Laser A4 Printers 					______	QTY: (  40    )

| Print Speed | 33 PPM |
| --- | --- |
| Resolution | Up to 1200x1200 dpi |
| Memory (RAM) | 128 MB Memory |
| Connectivity | USB 2.0 |
| Warranty | Three years warranty including Labor and spare parts |
| Others | All Drivers and Utilities should be included, USB Cable |

======================================================================

Item(5) : Color Laser A3 Printers 		            	   __  	    QTY: (   5   )

| Print Speed | 20 PPM (color A4) |
| --- | --- |
| Resolution | Up to 600x600 dpi (color) |
| Paper size | A3, A4, A5, B4, B5 |
| Memory (RAM) | 128MB Memory (Min.) |
| Connectivity | 1 Hi-Speed USB 2.0 |
| Warranty | Three years warranty including Labor and spare parts |
| Others | All Drivers and Utilities should be included, USB Cable |

==========================================================================

Item(6) : Dot Matrix Printer      			______		QTY: (   20     )

| Print Method | 24 Pin impact dot matrix |
| --- | --- |
| Character Spacing | 10, 12, 15, 17, 1, 20 |
| Resolution | 360x 360 |
| Connectivity | Hi-Speed USB 2.0 & parallel |
| Fonts | Draft & LQ ( Latin and Arabic multiple fonts) |
| Printable Width | 106 Columns @ 10 cpi |
| Copies | 1 original + 4 copies |
| Warranty | Three years warranty including Labor and spare parts |
| Others | All Drivers and Utilities should be included, USB Cable, Paper Feeder |

=================================================================================

Item(7): scanners

Item (7.1): ADF Scanner 			             ___      		      QTY (   10     )

| Scanner type | ADF (Automatic document feeder) |
| --- | --- |
| Scanning mode | Duplex / Simplex, Color / Grayscale / Monochrome |
| Scan resolution | Hardware: 600 x 600 dpi. Optical: 600 dpi |
| Scan speed | 20ppm/40ipm (300 dpi color, Gray, & B/W) |
| ADF Capacity | 50 sheets |
| Daily Cycle | 2000 Pages per Day |
| Interface | 1 x USB 2.0. |
| Power supply | 100 to 240 VAC 50/60 Hz |
| Other | AC Adapter, AC Cable, USB Cable, ,Driverand utilities . |
| Warranty | Three year warranty including Labor and spare parts |

Item (7.2 )   :  Flatbed Scanner                                                                QTY( 10)

|  | Feature | Minimum Specifications |
| --- | --- | --- |
| 1 | Scan type | Flatbed |
| 2 | Scan resolution | Up to 2400 dpi |
| 3 | Paper format | A4 |
| 4 | Compatible OS | Win10, win7 and win8.1 |
| 5 | Power supply | 100 ~ 240 VAC , 50-60 Hz. |
| 6 | other | Power cable, USB cable and software |
| 7 | Warranty | 3-years warranty including labor and spare parts |

===================================================================

Item(8): CCTV

Item (8.1): Indoor Dome Camera  _______      		      QTY (   120     )

· 1/2.8"-1/3" 3 Megapixel progressive scan CMOS

· Support H.265/H.264/MJPEG dual codec

· Smart triple streams supported

· Max 25/30fps@3MP(2048x1536)

· Supports PoE

· True WDR .

· DC Iris

· 3D DNR

· Defog

· Region of Interest : min. 3 zones

· Electronic Image Stabilizer (EIS)

· Smart detection

· Intelligent Function

· 2.8 ~12mm varifocal motorized lens

· Max. IR LEDs length 50m

· Micro SD memory

· Weatherproof: IP66

· Vandal Proof.

Item (8.2): Outdoor Bullet Camera                                                               QTY(2)

1/2.8-1/3” 3 Megapixel progressive scan CMOS

· Support H.265/H.264/MJPEG codec

· Max 25/30fps@3MP(2048x1536)

· Supports PoE

· True WDR .

· DC Iris

· Defog

· Region of Interest : min. 3 zones

· Electronic Image Stabilizer (EIS)

· Smart detection

· Ultra 3D DNR

· Intelligent Function

· Smart triple streams supported

· 2.8 ~12mm varifocal motorized lens

· IR Distance up to 50m

· Micro SD memory

· Weatherproof: not less than IP66

· Vandal Proof.

Item (8.3): Network Video Recorder                                            QTY    ( 1)

128 channels.

· H.265/H.264/MJPEG/MPEG4 codec decoding

· Max 320Mbps incoming bandwidth

· Up to 12Mp resolution preview&playback

· Max 16ch@1080P H.265/H.264 decoding

· 16 channel synchronous realtime playback, GRID interface

· Support Multi-brand network cameras: Arecont Vision, AXIS, Bosch, Brickcom, Canon, CP Plus, Dahua, Dynacolor, HIKvision, Honeywell, Panasonic, Pelco, Samsung, Sanyo, Sony, Videotec, Vivotek and etc.

· ONVIF Version 2.4 conformance

· Support 8 SATA HDDs up to 48TB, 4 USB(2 USB3.0, 2USB 2.0)

· Support RAID 0/1/5/6/10

· Minimum 2xHDMI/VGA simultaneous video output

· 3D intelligent positioning

· Multiple network monitoring

System must include enough storage for data for three months at least

===========================================================================

Item(9): switches & Transceivers

Item (9.1)	Aggregation  Switches                                               _      QTY(  2    )

Each switch must contain at least 24 ports SFP/SFP+ 1/10 GE Fiber

Each switch must contain at least 2 ports 40GE QSFP+

SFP+ tranceivers should be included

Performance up to 640 Gbps

Support the ability to manage the interconnected switches as a single logical device in a form of virtual chassis system

Layer 2 and 3 non-blocking ports

Wire-speed for IPv4 and IPv6 traffic and routing

Complete Layer 3 unicast and multicast routing protocol suites are supported, such as RIPv1/v2, OSPF, and multicast routing, including PIM-SM. All required licenses should be included.

All switch ports must be non-blocking without any oversubscription.

Redundant power supply on the offered solution

All needed fiber batch cord shall be provided

All Items shall be covered with Three Years minimum mother company warranty and support

Item (9.2) : 48 Ports Access Switches                                                      Qty (16)

Each Switch shall provide 48 x100/1000 Mbps RJ45 Ports. (POE+)

Each Switch shall provide 4 SFP/SFP+ 1G/10G Ports, all ports should be active.

Support the ability to manage at least four interconnected switches as a single logical device in a form of virtual chassis system

Should be fully managed switch through web and CLI.

Non-blocking architecture.

Switch Capacity: 176 Gbps at least.

Power over Ethernet power budget: 740 Watt

All needed fiber batch cord shall be provided

All Items shall be covered with Three Years mother company warranty and support

Item (9.3) : 24 Ports  Access Switches                                                     Qty (9)

Each Switch shall provide 24 x100/1000 Mbps RJ45 Ports. (POE+)

Each Switch shall provide 4 SFP/SFP+ 1G/10G Ports, all ports should be active.

Support the ability to manage at least four interconnected switches as a single logical device in a form of virtual chassis system

Should be fully managed switch through web and CLI.

Non-blocking architecture.

Switch Capacity: 128 Gbps at least.

Power over Ethernet power budget: 370 Watt

All needed fiber batch cord shall be provided

All Items shall be covered with Three Years mother company warranty and support

Item (9.4) SFP+ Transceivers

| description | QTY |
| --- | --- |
| 10G SFP+ Multimode Transceivers, active ports for edge switches and core | 44 |
| 10GSFP+single Mode Transceivers , active port For the new aggregation switch (EX-SFP-10GE-LR). | 4 |
| 10GSFP+single Mode Transceivers , active port For our existing JUNIPER core switch (EX-SFP-10GE-LR) in QAH data center | 4 |
| 10G SFP+ multimode Mode Transceivers, must be compatible with the existing nexus core in RMS data center | 2 |
| Multimode Fiber patch cord LC-LC (3 meters) | 2 |
| 10G fiber Ethernet for our existing server Fujitsu rx300s6 | 1 |

==============================================================================

Item(10) :	Wireless Access Points                                                         Qty (32)

Access points should be connected to the existing Aruba 7205 Controller in the Head Quarter.

Indoor Dual Radio and Dual Band Access Point.

Auto-sensing 10/100/1000 on the network port for 802.11ac APs.

The AP shall support MIMO with spatial streams 2x2:2SS and up to 1.3 Gbps wireless combined data rate.

The AP shall support Antenna gain minimum of 3 dBi in 2.4GHz and 6 dBi in 5GH with two integrated omni-directional antennas.

Support 802.3af standard Power-over-Ethernet (PoE) with full capacity operation at full power of the radios – and 2 spatial streams for the 802.11ac capable APs

Support the use of 802.11ac Wave 1 and MIMO technologies on 2.4GHz radios and 5GHz

Multi-service concurrent 802.11a/n + b/g/n/ac WLAN

Automatic transmit power and channel management control with auto coverage hole correction via Adaptive Radio Management (ARM) or CleanAir.

The Access point shall support complete configuration and control through the centralized Wireless LAN Controller.

The Access point shall be completely plug and play requiring no manual configuration. It shall automatically self configure from a single central point.

The Access Point shall have a console port for easy management and troubleshooting.

The Access Point shall have Wall and Ceiling Mouting Kit.

The Access Point shall have license to operate and run on the contrller

All Items shall be covered with Three Years mother company warranty and support

=====================================================================

Item(11) :	Aruba 7205 Wireless LAN Controller Controller              Qty(1)

Provide HA functionality for the current WLC in QAH data center

=============================================================================

Item(12) :	Workstation _                                                                    QTY(  4    )

| Processor | 2 X Intel Xeon E3-1225 v5 (3.3 GHz, 8 MB cache, 4 cores) |
| --- | --- |
| Chipset | Intel C236 |
| Memory | 8GB DDR4-2133 ECC SDRAM- 4 DIMM Slots |
| Networking | Intel 10GbE NIC, Dual Port, |
| Video Card | Intel HD Graphics P530 |
| Expansion Slots | 1 PCIe Gen3 x16 |
|  | 1 PCIe Gen3 x4 (x4 connector) |
|  | 1 PCIe Gen3 x4 (x16 connector) |
|  | 1 PCIe Gen3 x1 |
|  | Must support all below cards: |
|  | - NVIDIA Quadro K420 (2 GB) - NVIDIA NVS 310 (1 GB); - NVIDIA NVS 315 (1 GB); - NVIDIA NVS 510 (2 GB) - AMD FirePro W2100 (2 GB); - NVIDIA Quadro K420 (2 GB); - NVIDIA Quadro K620 (2 GB) - NVIDIA Quadro K2200 (4 GB); - AMD FirePro W5100 (4 GB) - AMD FirePro W7100 (8 GB); - NVIDIA Quadro M4000 (8 GB) |
| Storage | 2 ×500 GB SATA/ 7200 rpm RAID 1 |
| Optical Storage | DVD +/-RW DL Super Multi |
| USB Ports | 5 USB Ports & 1 DVI-I single link |
| Audio | Integrated Audio card |
|  | Stereo Headphone/Microphone combo jack |
| Display | Min 23” LED screen professional display |
| Security | Chassis lock slot |
| Brand/Country of origin | USA, Europe, or Japan Only Or, Company is one of the top 5 Workstations Market share leaders in 2015 |
| Warranty | 5 Years Hardware Service with 5 Years Next Business Day Onsite Service. |
| Installation Services | Hardware and Software Onsite installation service |
| Others | System Unit, Chassis, Motherboard, Monitor, Mouse and Keyboard are Same Brand |

=======================================================================

الشروط الخاصة :

- كافة المواد المقدمة يجب ان تكون جديدة 100%   وان يكون تــــــــاريخ الصنع ليس قبل عام 2016.

- أن تكون المواد معفاة من كافة الرسوم الجمركيه والضريبه العامه على المبيعات واية رسوم او عوائد اخرى

- تقديم ورشة عمل لمدة لا تقل عن اسبوع لتنصيب نظام التشغيل على الخادم في البند (1) اعلاه وتدريب (3 ) اشخاص على الاقل ولمدة اسبوع .

- يجب على الشركة بيان بلد المنشأ للأجهزة المعروضة.

- يسمح بتجزئة العطاء حسب البنود على ان يكون البند (item) كامل .

- الاسعار المقدمه من قبل  الشركة المحال عليها البند رقم (1) شامل التركيب والتشغيل وتنصيب البرامج التطبيقية العاملة على خوادم مركز البيانات واي برامج اخرى لازمة للتشغيل بما فيها (data migration ) بدون اي تكلفة مالية جديدة على الخدمات الطبية الملكية.

- تلتزم الشركة بتزويد القوات المساحة الاردنية بقطع الغيار اللازمة ولمدة عشر سنوات,  بعد انتهاء الكفالة المجانية , مقابل الثمن حسب الاسعار السائدة في حينه.

- على الشركة توضيح كل مواصفة من مواصفات العطاء بعرضها وارفاق ما يثبت ( كتالوجات وعرض الشركة ) بأن المواصفة متوفرة بالاجهزة المقدمة و تقديم Compliant Sheet مع ذكر رقم الصفحة بالكتالوج و العرض , مقابل كل مواصفة

- تتعهد الشركة بالقيام بزيارات صيانة وقائية ربعية في مواقع الاجهزة خلال فترة الكفالة المجانية .

- يجب بيان نوع وموديل الجهاز المقدم  .

- بيان مدة الكفالة المجانية لجميع البنود اعلاه .

- تسليم وتركيب المواد كافة في الخدمات الطبية الملكية في الموقع الذي تقرره دائرة تكنولوجيا المعلومات الطبية .

- تلتزم الشركة المحال عليها العطاء بتوفير لاصق (Label) على جميع الاجهزة مبينا علية : اسم الشركة , تاريخ الاستلام , تاريخ انتهاء الكفالة المجانية , الرقم المتسلسل و رقم العطاء.

- في حال استبدال القرص الصلب  اثناء فترة الكفالة المجانية يتم استبداله بدون بديل وتحتفظ الخدمات الطبية الملكية بالقرص التالف.

- على الشركة بيان كافة تفاصيل التدريب والاسعار مفصلة  .

- تلتزم الشركة المحال عليها العطاء بتنزيل كافة البرمجيات المطلوبة من ضمن العطاء .

- يجب ان تكون الاجهزة وكافة تفرعاتها اصلية BRAND NAME وصالحة ولن تقبل العروض بالاجهزة البديلة  او اجهزة التجميع .

- تلتزم الشركة المحال عليها  بند ما او اكثر  بتقديم كامل المطلوب في البند شامل التركيب والتشغيل (turn-key solution ) بدون ان تتحمل الخدمات الطبية الملكية اي التزامات مالية جديدة .

Annex D

الاثاث غير الطبي لمشروع مركز معالجة الاورام

مواصفات الأجهزة الكهربائية

Item D1:

Under counter Refrigerator						Qty. (80)

- General Description

Under counter refrigerator without freezer.

- Construction

Metallic body with high quality exterior paint.

- Overall Dimensions(Approximate)

Width:		400mm

Depth:		400mm

Height:		600mm

- Finishes

Colors to be selected from manufacturer's standard range, utilizing highest quality within thatrange (where applicable).

- Special Features

Design and Construction to be anti-bacterial and anti-fungal.

ItemD2:

Domestic Refrigerator						Qty. (30)

- General Description

Domestic house-hold refrigerator with freezer compartment with its own separate external door.

- Construction

Metallic body with high quality exterior paint.

- Overall Dimensions(Approximate)

Width:		550mm

Depth:		550mm

Height:		1500mm

- Finishes

Colors to be selected from manufacturer's standard range, utilizing highest quality within thatrange (where applicable).

- Special Features

Design and Construction to be of the no frost type.

ItemD3:

Copy Machine- Desktop Model					Qty. (15)

General Description

Monochrome Copying Black and White, Desktopmodel. Training video.    Average+/- 35 copies per minute. Standard Options included. 60-sheet Duplex automatic feeder.

Submit and Coordinate with the End User for all requirements prior to final pricing.

- Overall Dimensions (Approximate)

Width:		582mm

Depth:		488mm

Height:		553mm

- Electrical Requirements

Volt:		240V

Hz:			50/60

- Finishes

Manufacturer’s standard.

- Fire Retardant

Construction to comply with Civil Defense/Local regulations or the equivalent.

ItemD4:

Copy Machine, Floor model					Qty. (10)

- General Description

Monochrome Copying Black and White, floor model with 100-sheet-circulating document handler. Training video. Average+/- 60 copies per minute. Standard Options included. 4-tray option with built in bulk feeder (total 5 trays).

Submit and Coordinate with the End User for all requirements prior to final pricing.

- Overall Dimensions Chair

Width:		955mm

Depth:		650mm

Height:		1140mm

- Electrical Requirements

Volt:		240V

Hz:			50/60

- Finishes

Manufacturer’s standard.

- Fire Retardant

Construction to comply with Civil Defense/Local regulations or the equivalent.

ItemD5:

Led TV, 32 inch							Qty. (115)

Type:LCD

Color:Black

Screen Size:  32 inch

Inputs & Outputs: (HDMI: 2) (USB: 1)

ItemD6:

Led TV, 40 inch							Qty. (10)

Type:     LCD

Color:    Black

Screen Size:  40 inch

Inputs & Outputs: (HDMI: 2) (USB: 1)

الشروط الخاصة:

- أن تكون جميع المواد الموردة جديدة وصالحة 100% وخالية من أي عيوب في الصنع أوفي المادة ومن طرازحديث لم يتوقف إنتاجها (القديم يرفض).

- لا مانع من تجزئة المواد.

- على المتقدمين للعطاءتقديم استعلاماتهم خلالال الثلث الاول من فترة اغلاق العطاء.

- تقييم المناقص حسب الكفاءة والملاءة المالية.

- ان تحتوي العروض على كفالة مصنعية لمدةعامين.

- تقديم الكتيبات أو(DVD/CD)يحتويعلى(Operation Manual) و (Service Manual) عند التسليم.

- الأسعارشاملة الشحن،النقل الى الموقع،التركيب ومايحتاج من تأسيسات لضمان التركيب والتوصيلات الكهربائية والتشغيل على أكمل وجه بمافي ذلك ربط شاشات التلفزيون مع نظام الاستقبال المركزي وحسب الوضع القائم.

- مكان التسليم والتركيب والتشغيل في مركز معالجة الاورام وحسب تعليمات لجان الاستلام وشعبة الاثاث واللوازم غير الطبية.

- أن تعمل الأجهزة الكهربائيةعلى فولتية  (240V, 50/60Hz) للموادالكهربائية.

- شاشات التلفزيون: ان تكون المداخل والمخارج جانبية.

- مدةالتسليم خلال (120) يوم من التبليغ بالإعفاءالجمركي، وفي حال عدم استخدام كتاب الإعفاءالجمركي تكون مدةالتسليم خلال (120يوم) من تاريخ التبليغ بالإحالة النهائية.

- تقديم جميع البروشورات الفنية.

- تقديم شهادة منشأ للمواد.

- تقديم كفالة صيانة بنكية بقيمة 5% من إجمالي قيمة الإحالة للموادشاملة قطع الغيارالعاملة صالحة لمدة سنتين من تاريخ التركيب والتشغيل.

- تقديم تعهد بتوفيرقطع الغيارلمدة 5 سنوات من تاريخ التركيب والتشغيل للمواد.

- تقديم قائمة مسعرة بالقطع الاحتياطية صالحة لمدةخمس سنوات بعد انتها ءكفالة الصيانة للمواد.

Annex E                                                                                                                                      مواصفات خزائن سجل المرضى

المواصفات الفنية لخزائن سجل المرضى لمركز معالجة الاورام

- تقديم وتوريد وتركيب خزائن لسجل المرضى في مشروع مركز معالجة الاورام

- الخزائن تتحرك بواسطةالكهرباء ( كبسةالكترونية مع اضاءة مثبته عل ىالخزانة مع شاشة ديجتال).

- امكانيةالبحث عن الملفات في نفس الخزانة من الطرفين.

- وجودنظام حماية حساس Sensor و Leg Barلمنع تحريك ا لخزانة في حال وجود اشخاص بين الخزائن

- وجود حساس  Sensorلمعرفة درجة الحرارة والرطوبة.

- وجود كاشف دخان لكل خزانة.

- وجو د نظام اضاءة علوية لكل خزانة خلال الاستخدام .

- الخزانة ا لواحدة تسيرعلى نظام السكك وسهلة الانزلاق.

- تحريك الخزانة يدويا اوبواسطة يد التحريك بدون كهرباء.

- في حالة استعمال يد التحريك اليدوي يتم انزلاق الخزانة بسهولة وبمعزل عن ماتور الكهرباء.

- يوجد في كل خزانة 40 رف تقريبا.

- مساحة التحريك بين الخزائن من 80-100 سم وحسب حجم المستودع.

- دهان ا لخزائن من الخارج والداخل معالج حراريا والزوايا الداخلية تتغير.

- سهولة فك وتغييرحجم الرف حسب حجم الاضبارة.

- وزن المتر الواحد شامل الخزانة والملفات 160 كغم  وان يتم اضافة  سكك اضافية في حالة ضعف الارضية لتوزيع الاحمال على مساحة الغرفة.

- ان تكون متانة السكك عالية الجودة من الستانلس ستيل والقاعدة حديد معالج حراريا.

- تقدم العروض بناء  على زيارة المواقع المخصصة للسجلات في مركز معالجة الاورام ودراسة المساحات المتوفرة.

الشروط:

- على المتقدمين للعطاء تقديم استعلاماتهم خلال الثلث الاول من فترة اغلاق العطاء .

- ان تحتوي العروض على كفالة مصنعية لمدةعاميين ( سنتين ).

- تقديم الكتيبات اوDVD/CD يحتوي على ( Operational manual  )و ( service manual ) عند التسليم.

- الاسعار شاملة الشحن , النقل الى الموقع , التركيب ومايحتاج من تاسيسات لضمان التركيب والتشغيل على اكمل وجه.

- مكان التسليم والتركيب والتشغيل في مركز معالجة الاورام وحسب تعليمات لجان الاستلام وشعبة الاثاث واللوازم الغيرطبية.

- مدةالتسليم خلال (120) يوم من التبلغ بالإعفاء الجمركي وفي حال عدم إستخدام كتاب الإعفاء الجمركي تكون مدةالتسليم خلال (120يوم) من تاريخ التبلغ بالإحالة النهائية.

- تقديم جميع ا لبروشورات الفنية.

- تقديم شهادة منشأ للمواد.

- تقديم شهادات الجودة ا لمصدقة اللازمة للمواد عند التسليم.

- تقديم صورة مصدقة عن شهادة المنشأ عند التسليم.

- تقديم كفالة صيانة بنكية بقيمة 5% من إجمالي قيمة الإحالة للمواد شاملة قطع الغيارالعاملة صالحة لمدة سنتين من تاريخ التركيب والتشغيل.

- تقديم تعهد بتوفير قطع الغيار لمدة 5 سنوات من تاريخ التركيب والتشغيل للمواد.

- تقديم قائمة مسعرة بالقطع الإحتياطية صالحة لمدة خمس سنوات بعد إنتهاء كفالة الصيانة للمواد.

- تقديم ( compliance shat  ) تحتوي على خانة تشير الى البرشور ويحق للجنة الشراء في حال عدم المطابقة , الرفض الجزئي اوالكلي علماً بأن المناقصيين مسؤولين عن صحة  (compliance sheet  ) , الاخطاء والمعلومات التضليلية تقود الشركة الى وضعها على القائمة السوداء مستقبلياً .

- التعهد بتدريب العاملين في مركز معالجة الاورام على استخدام وتشغيل النظام.

- على المناقصيين زيارة موقع تركيب النظام في مركز معالجة الاورام وتحديد اي معوقات تحول دون التركيب وبخلاف ذلك تتحمل الشركة مسؤولية تصويب اي عائق مستقبلا.

Annex F

مواصفات أجهزة الدور الالي و الانتظار

QUEUE MANAGEMENT SYSTEMS

| No | Description | Qty() |
| --- | --- | --- |
| 1. | Touch screen ticket dispenser, with metal stand epoxy powder coated جهاز طباعة التذكرة مع قاعدة حديد مطلي حراري | 7 |
| 2. | Counter display 3-Digit dot matrix high resolution شاشة الموظف أو الشباك تظهر رقم العميل مع اظهار رقم الشباك او رمز الخدمة مع رسالة متحركة | 30 |
| 3. | Counter terminal including: all function, keys, open, closed, next, cal, store, transfer. قاعدة الموظف مع خدمة التحويل الى موظف آخر و خدمات أخرى. | 30 |
| 4 | MAIN DISPLAY WAITING AREADOT-ATRIX تظهر رقم العميل مع سهم لتحديد الاتجاه ونوع الخدمة حرفا ورقم الشباك مع إظهار رسالة متحركة SCROLLING MESSGE باللغة العربية والانجليزية على إن تكون الشاشات حسب طبيعة أماكن الانتظار وذات خط واحد أو شاشه ذات خطان أو شاشه ثلاثة خطوط أو شاشه أربعة خطوط في لوحه واحدة مع إمكانية مع إمكانية تثبيت الشاشة على الحائط. يمكن مشاهدتها من مسافة 250 م والإطار الخارجي مصنع إما البلاستيك أو الألمنيوم أو الستانليس . | 20 |
| 5 | Accessories (communication card, distributor, amplifier, supplies and all necessary and extra power supply) شامل التركيب و القطع و الوصلات و حوامل الشاشات و الموزع الرئيسي و السماعات |  |
| 6. | Paper Roll Ticket 20 roll (15000 ticket) | 50 SET |
| 7. | Software QUEUE MANAGEMENT SYSTEMS In Hospital | 7 |

الشروط

- الاسعارشاملة الشحن , النقل الى الموقع , التركيب ومايحتاج من تاسيسات لضمان التركيب والتشغيل على اكمل وجه.

- التسليم و التركيب و التشغيل في مركز معالجة الاورام /الخدمات الطبية الملكية اوحسب تعليمات شعبة الاثاث واللوازم غير الطبية.

- تلتزم الشركة بتدريب فنيين و لأي عدد ممكن و مدة غير محدودة

- تقديم شهادة منشأللمواد.

- تقديم شهادات الجودة المصدقة اللازمة للمواد عند التسليم.

- تقديم صورة مصدقة عن شهادة المنشأعندالتسليم

- تقديم تعهد بالالتزام بالأسعار الواردة و أسعار الورق.

- تلتزم الشركة بصيانة مجانية للاجهزة و لمدة عامين.

- تقديم الكتيبات اوDVD/CD يحتوي على ( Operational manual  )و ( service manual ) عند التسليم.

Annex G

مواصفات اللوحات الارشادية

مواصفات اللوحات الارشادية

- Item G-1تقديم وتركيب لوحات ارشادية لمركز معالجة الاورام مكونة من الواح اكرليك سماكة 3 ملم عليه ستيكر مطبوع حسب التصميم المعتمد من لجنة الشراءويتم تركيب اللوحة على طبقة خلفية من الالمنيوم سماكة 4 ملم لون فضي ويتم تثبيت اللوحة بواسطة براغي عليها اغطية ستانليس قطر 16 ملم وحسب القياسات والاعدادا التالية:

| النوع | اللوحة | القياس (سم) | العدد المطلوب (دراسة اولية) |
| --- | --- | --- | --- |
| A | دليل رئيسي | 220*180 | 3 |
| B | دليل فرعي | 100*80 | 22 |
| C | لوحة غرفة ادارية | 30*12 | 200 |
| D | لوحة ارقام غرف لمرضى | 20*20 | 160 |
| E | لوحة غرف خدمات مع رمز | 20*20 | 170 |
| F | لوحة اقسام معلقة | 60*15 | 100 |

Item G-2 – تقديم وتركيب احرف مجسمة من جنب المنيوم ووجه اكريليك سماكة 5 ملم عليه ستيكر ثنائي اللون اسود نهارا وابيض ليلا ومضاء من الداخل بواسطة وحدات LED وحسب الاسم الذي سيتم اعتماده للمركز وكمايلي:

| النوع | القياس |
| --- | --- |
| A2 | الواجهة الشرقية (الاسم المعتمد للمركز)عربي فقط بطول اجمالي 17 متر |
| B2 | الواجهة الجنوبية المقابلة لمستشفى الملكة علياء العسكري (الاسم المعتمد للمركز)عربي فقط بطول اجمالي 7 متر |

الشروط الخاصة:

- الاسعارشاملة الشحن , النقل الى الموقع , التركيب ومايحتاج من تاسيسات لضمان التركيب والتشغيل على اكمل وجه.

- التسليم و التركيب و التشغيل في مركز معالجة الاورام /الخدمات الطبية الملكية اوحسب تعليمات شعبة الاثاث واللوازم غير الطبية

- تقديم مخططات تنفيذية لمواقع اللوحات على الواقع بالإضافة إلى تصميم كافة اللوحات حسب إختيار لجنة الشراء.

- يتم تزويد الشركة المنفذة للوحات بالمسميات المطلوب طباعتها على اللوحات من قبل لجنة الشراء .

- يتم تقديم عينات مقترحة للون ونوع وحجم الخط ليتم إعتمادها من لجنة الشراء .

- على الجهه المنفذة معاينة مواقع التركيب وبيان اي عوائق وإعادة الوضع كما كان علية قبل التركيب وخلاف ذلك تتحمل الشركة مسؤولية معالجة اي عوائق تحول دون التركيب على الوجه الامثل.

- تتحمل الشركة المنفذة مسؤولية أي أضرار تلحق بمواقع التركيب وإعادة الوضع كما كان عليه.

- الكميات المدرجة بالكشف المرفق تقريبية وسيتم المفاضلة ماليا على هذا الاساس ويتم إعتماد السعر الأفرادي لأي زيادة ونقصان لكافة البنود وحسب مايتم اعتماده من لجنة الشراء ولجان الإستلام .

- تقديم كفالة صيانة لمدة عامين من تاريخ الاستلام

- تقديم وتركيب الأرمة الرئيسية لإسم المركز حسب التسمية المعتمدة والمواصفات التالية :

- احرف مجسمة من جنب المنيوم أكرليك سماكة 5ملم مغطى بمادة (dual . Film color) أبيض ليلاً وأسود نهارا.

- خلفية الحرف ألواح (pvc) فوم سماكة 20 ملم .

- إضاءة الأحرف (led module) لون أبيض منشأ كوري أو أمريكي مكفول مدة لاتقل عن خمس سنوات مع المحولات والتوصيلات وكل مايلزم لإتمام العمل بالشكل المطلوب.

Annex H

مواصفات أثاث المختبرات

الاثاث غير الطبي لمشروع مركز معالجة الاورام

Technical Specifications for laboratory furniture

| ITEM # | ITEM DESCRIPTION |  | UNIT | ELEV. | QTY |
| --- | --- | --- | --- | --- | --- |
| Building (A): Second Floor |  |  |  |  |  |
| 1.1 | Wall Bench, size:2500x900x900 mm, consisting of: |  | No. | 1 | 1 |
|  | Qty. | Description |  |  |  |
|  | 2.5 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 1 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 2 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 2 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 3 no. | Double C-frame side with frame connections. |  |  |  |
| 1.2 | Wall Bench, size: 3050x900x900 mm, consisting of: |  | No. | 2 | 1 |
|  | Qty. | Description |  |  |  |
|  | 3.05 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:600x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:600x500x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size:550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 2 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 1 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
| 1.3 | Wall Bench, size:1975x900x900 mm, consisting of: |  | No. | 3 | 1 |
|  | Qty. | Description |  |  |  |
|  | 1.98 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench Door & Drawer cabinet, size:600x500x600mm. |  |  |  |
|  | 1 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 1 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 2 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 3 no. | Double C-frame side with frame connections. |  |  |  |
| 1.4 | Wall Bench, size:3050x900x900 mm, consisting of: |  | No. | 4 | 1 |
|  | Qty | Description |  |  |  |
|  | 3.05 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:600x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:600x500x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size:550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 2 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 1 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 3 no. | Double C-frame side with frame connections. |  |  |  |
| Building (A): Fifth Floor |  |  |  |  |  |
| 2.1 | Wall Bench, size:1550x900x900 mm, consisting of: |  | No. | 1 | 1 |
|  | Qty. | Description |  |  |  |
|  | 1.55 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:600x500x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size:550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 1 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 1 no. | Overhead cabinet, with 1 door, size: 600x300x600mm. |  |  |  |
|  | 3 no. | Double C-frame side with frame connections. |  |  |  |
| Building (B): First Floor |  |  |  |  |  |
| 3.1 | Wall Bench, size:4940x900x900 mm, consisting of: |  | No. | 1 | 1 |
|  | Qty. | Description |  |  |  |
|  | 4.94 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:600x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench Door & Drawer cabinet, size:600x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size:550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 1 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 2 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 3 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 5 no. | Double C-frame side with frame connections. |  |  |  |
| Building (C): Ground Floor |  |  |  |  |  |
| 4.1 | Wall Bench, size:5755x900x900 mm, consisting of: |  | No. | 1 | 1 |
|  | Qty. | Description |  |  |  |
|  | 5.76 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size: 600x500x600mm. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size: 450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:600x500x600mm. |  |  |  |
|  | 2 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 2 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 2 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 4 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
| 4.2 | Wall Bench, size:4160x900x900 mm, consisting of: |  | No. | 2 | 1 |
|  | Qty. | Description |  |  |  |
|  | 4.16 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size:550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 1 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 1 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 5 no. | Double C-frame side with frame connections. |  |  |  |
| 4.3 | Wall Bench, size:5865x900x900 mm, consisting of: |  | No. | 3 | 1 |
|  | Qty. | Description |  |  |  |
|  | 5.86 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:600x500x600mm. |  |  |  |
|  | 2 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 2 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 2 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 4 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 8 no. | Double C-frame side with frame connections. |  |  |  |
| Building (C): Ground Floor – Plasmapheresis |  |  |  |  |  |
| 5.1 | Wall Bench, size:4185x900x900 mm, consisting of: |  | No. | 1 | 1 |
|  | Qty. | Description |  |  |  |
|  | 4.185 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size: 450x500x600mm. |  |  |  |
|  | 2 no. | Under-bench 2 Doors cabinet with adj. shelf, size: 900x500x600mm. |  |  |  |
|  | 3 no. | Overhead glass 2 doors cabinet with wooden frame, size: 900x300x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size:550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 1 no. | Polypropylene Pegboard size:550*700mm. |  |  |  |
|  | 1 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 3 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 6 no. | Double C-frame side with frame connections. |  |  |  |
| Building(C): First Floor |  |  |  |  |  |
| 5.3 | Wall Bench, size:3625x900x900 mm, consisting of: |  | No. | 1 | 1 |
|  | Qty. | Description |  |  |  |
|  | 3.63 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size: 450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size: 450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 2 Doors cabinet with adj. shelf, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size: 550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 1 no. | Laboratory bench mounted eye wash. |  |  |  |
|  | 1 no. | Polypropylene Pegboard size:550*700mm. |  |  |  |
|  | 2 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 3 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 1 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 5 no. | Double C-frame side with frame connections. |  |  |  |
| 5.4 | Storage Tall cabinet, size:900x500x2120 mm |  | No. | 2 | 2 |
| Building (C): Second Floor - LAB (1) |  |  |  |  |  |
| 6.1 | Wall Bench, size:3720x900x900 mm, consisting of: |  | No. | 1 | 1 |
|  | Qty. | Description |  |  |  |
|  | 3.72 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 2 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 2 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 2 no. | Overhead glass 2 doors cabinet with wooden frame, size: 900x300x600mm. |  |  |  |
|  | 2 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 3 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 5 no. | Double C-frame side with frame connections. |  |  |  |
| 6.2 | Wall Bench, size:7372x900x900 mm, consisting of: |  | No. | 2 | 1 |
|  | Qty. | Description |  |  |  |
|  | 7.37 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 2 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 2 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:600x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 3 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 3 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 6 no. | Double C-frame side with frame connections. |  |  |  |
| 6.3 | Wall Bench, size:5450x900x900 mm, consisting of: |  | No. | 3 | 1 |
|  | Qty. | Description |  |  |  |
|  | 5.45 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 2 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 2 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 4 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 2 no. | Overhead glass 2 doors cabinet with wooden frame, size: 900x300x600mm. |  |  |  |
|  | 3 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 4 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 6 no. | Double C-frame side with frame connections. |  |  |  |
| 6.4 | Wall Bench, size:3175x900x900 mm, consisting of: |  | No. | 4 | 1 |
|  | Qty. | Description |  |  |  |
|  | 3.17 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 2 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size:550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 1 no. | Laboratory bench mounted eye wash. |  |  |  |
|  | 1 no. | Polypropylene Pegboard. |  |  |  |
|  | 1 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 4 no. | Double C-frame side with frame connections |  |  |  |
| Building (C): Second Floor - LAB (2) |  |  |  |  |  |
| 7.1 | Wall Bench, size:9678x900x900 mm, consisting of: |  | No. | 1 | 1 |
|  | Qty. | Description |  |  |  |
|  | 9.6 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 3 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 3 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 3 no. | Under-bench 1 Door cabinet with adj. shelf, size:600x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 2 Doors cabinet with adj. shelf, size:900x500x600mm. |  |  |  |
|  | 4 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 4 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 11 no. | Double C-frame side with frame connections. |  |  |  |
| 7.2 | Wall Bench, size:3705x900x900 mm, consisting of: |  | No. | 2 | 1 |
|  | Qty. | Description |  |  |  |
|  | 3.7 m. | 16mm thick Compact Laminate Worktop, 900mm Width. |  |  |  |
|  | 1 no. | Under-bench Drawer cabinet, size:450x500x600mm. |  |  |  |
|  | 2 no. | Under-bench 1 Door cabinet with adj. shelf, size:450x500x600mm. |  |  |  |
|  | 1 no. | Under-bench 1 Door cabinet with adj. shelf, size:600x500x600mm. |  |  |  |
|  | 1 no. | Sink cabinet, with 2 doors for sink, size: 900x500x600mm. |  |  |  |
|  | 1 no. | Polypropylene Sink, Size:550x450x310mm with polypropylene bottle trap. |  |  |  |
|  | 1 no. | Laboratory bench mounted one-hole water mixer. |  |  |  |
|  | 1 no. | Laboratory bench mounted eye wash. |  |  |  |
|  | 1 no. | Polypropylene Pegboard. |  |  |  |
|  | 1 no. | 2- Single electrical socket with 2- Single Pyramid plastic holder. |  |  |  |
|  | 2 no. | Overhead cabinet, with 2 doors, size: 900x300x600mm. |  |  |  |
|  | 1 no. | Overhead glass 2 doors cabinet with wooden frame, size: 900x300x600mm. |  |  |  |
|  | 1 no. | 600mm High Knee Space Filler. |  |  |  |
|  | 4 no. | Double C-frame side with frame connections. |  |  |  |

Laboratories Specifications

- General

The laboratory furniture manufacturer must have a long-standing experience in the manufacturing and installation of laboratory furniture and fume Hoods. It Is important that the supplier have full knowledge of the environments, use and performance requirements. The laboratory manufacturer should be certified with the international quality system ISO 9001-2008.

The manufacturer shall be responsible for supplying all items under the scope of this section.

The Laboratory manufacturer shall submit the following documents/material for approval:

- Reference with names and locations where laboratory furniture has been supplied and installed.

- Complete and detailed technical specifications of material.

- Complete set of drawings including furniture layout, elevations, sectional views, and details.

- Complete and full set of samples covering all material used including but not limited to worktops, sinks, bottle traps, pegboards, electro-mechanical fittings, cabinets components/materials, proposed colour scheme samples, etc.

- Manufacturer warranty on materials, manufacturing, and installation.

- Any other documents or material as requested by the Consultant.

- Laboratory Components:

- Bench System

The suspended bench system is supported on heavy-duty steel frames fully carrying the worktops. Under-bench cabinets are suspended on the frames and can be relocated with simple hand tools. All sides and middle frames are constructed without a vertical front leg to avoid obstruction for the user. Frames are constructed from 60*30*1.8 mm rectangular steel section. Legs are supplied with adjustable feet ±15 mm to correct eventual unevenness of the floor. All frame-work are pre-treatment epoxy powder coated.

- Worktops

Worktops shall be Made of Maica's mailab Chem Compact laminate lab grade 16mm thick is a high pressure decorative compact laminate manufactured especially for the chemical laboratory environment through a process of pressing layers of assembled papers under high pressure and heat to form solid sheets. These solid sheets are impact, wear, and heat and corrosion resistant, do not support bacteria (hygienic) and have resistance to cigarette burns, boiling water and stains. With high resistance to chemicals.

Benches shall be supplied with 100 mm back splash on walls made of the same top material.

Worktops shall be supplied with a continuous drip groove 3 mm wide and 3 mm deep on the underside of the edges.

- Sinks

Laboratory benches shall be supplied with black Laboratory grade polypropylene sink with polypropylene bottle trap.

Material: High grade PP, resistant to most chemicals 
Accessories: High grade PP drainer, PP basket trap & PP drainer cover.

Two sizes:

- Inside: 490*390*290 Outside: 550*450*310.

- Inside: 380*270*260 Outside: 430*320*270.

- Water Fittings

Fitting shall be manufactured according to standard EN13792:2000, from best quality (DIN 12898).

Fitting shall be surface-treated with highly chemical resistant powder.

Handles shall be supplied with text symbols indicating media type such as GAS. CW.HW.

Maximum working pressure for water fitting shall be 10 bars.

Neck reach of water fitting shall be 150mm minimum.

- Cabinets

Cabinets shall be made of 18 mm plywood covered with 0.7 mm HPL (Formica) on all faces with PVC edging strips.

Shelves shall be mounted on heavy duty shelf supports adjustable at regular height of 32 mm and edged on four sides with PVC.

Doors and drawer fronts shall be made of 18mm plywood with 0.9mm HPL on both sides with 3 mm rounded PVC edging strips. 0.9 mm HPL colour and 3 mm PVC colour shall be as per the Consultant choice and approval.

Each door shall be mounted with Snap-On hinges with 110 opening angles.

Drawers bottom shall be made of 12 mm covered with 0.7 mm thick HPL.

Drawer sides and slides shall be made of epoxy powder-coated steel running on nylon runners.

Doors and drawers shall be supplied with flush length grab-rail made of epoxy powder-coated anodized aluminium profile.

Back of sink cabinets shall be made of 18mm plywood (200 mm height) covered with 0.7 mm thick HPL on both sides & PVC edging strips.

Wall cabinets shall be huge using 2 hanging fittings, adjustable in 3 dimensions.

- Paneling

Panels shall be made of 12 mm plywood and covered HPL on both sides with PVC edging strips in same colour on all edges.

- Wall shelving units.

Shelves shall be made of 18 mm thick plywood covered with HPL on both sides with PVC edging strips.

- Electrical Fittings

Socket outlets shall be 240v, 13 amps. Double and single pole switching mechanism on switched sockets comply with BS 1363 Pt 2, A.C. only.

Electrical socket standout/holder shall be made of epoxy powder-coated steel or brass with flange and nut connection on the worktop.

- Pegboard

Pegboards are made of polypropylene material for easy cleaning, structural rigidity, light weight, and easy assembly. Pegboards are supplied with draining hose, sloping drip tray that makes directing liquid easier and quicker. 56 pegs of length 700*500mm.

الشروط الخاصة:

- الأسعار شاملة الشحن، النقل الى الموقع، التركيب وما يحتاج من تأسيسات لضمان التركيب والتوصيلات الكهربائية والتشغيل على أكمل وجه.

- مكان التسليم والتركيب والتشغيل في مركز معالجة الاورام وحسب تعليمات لجان الاستلام وشعبة الاثاث واللوازم غير الطبية.

- تتحمل الشركة المنفذة مسؤولية ربط الاثاث المختبري على مصادر الكهرباء والمياه والصرف الصحي.

- تقديم مخططات تنفيذية لمواقع الاثاث على الواقع ويتم اخذ موافقة مديرية الخدمات الطبية على المخططات.

- على الشركات المناقصة معاينة مواقع التركيب وبيان اي عوائق وإعادة الوضع كما كان علية قبل التركيب وخلاف ذلك تتحمل الشركة مسؤولية معالجة اي عوائق تحول دون التركيب على الوجه الامثل.

- تتحمل الشركة المنفذة مسؤولية أي أضرار تلحق بمواقع التركيب وإعادة الوضع كما كان عليه.

- بيان أقرب مدة للتسليم.

- تقديم جميع البروشورات الفنية.

- تقديم شهادة منشأ للمواد.

- تقديم شهادات الجودة المصدقة اللازمة للمواد عند التسليم.

- تقديم صورة مصدقة عن شهادة المنشأ عند التسليم.

- تقديم كفالة صيانة بنكية بقيمة 5% من إجمالي قيمة الإحالة للمواد شاملة قطع الغيار العاملة صالحة لمدة سنتين من تاريخ التركيب والتشغيل.

- تقديم تعهد بتوفير قطع الغيار لمدة 5 سنوات من تاريخ التركيب والتشغيل للمواد.

- تقديم قائمة مسعرة بالقطع الاحتياطية صالحة لمدة خمس سنوات بعد انتهاء كفالة الصيانة للمواد.

- تقديم (Compliance Sheet) تحتوي على خانة تشير الى البروشور، ويحق للجنة الشراء في حال عدم المطابقة، الرفض الجزئي او الكلي علماً بأن المناقصين مسؤولين عن صحة (Compliance Sheet) الاخطاء والمعلومات التضليلية تقود الشركة الى وضعها على القائمة السوداء مستقبلًا.

Annex I

مواصفات اجهزة معالجة النفايات الطبية

Medical Waste Equipment Specifications

General Specifications for Medical Waste Management for Oncology center

Bidders are required to submit their solutions to handle medical waste of the oncology center. The offered system must handle a minimum of 800 kg of medical waste for an eight-hour daily shift; Due to the nature and volume of the medical waste, a modular solution (not more than two units) may be required.

System Description:

- General:

- The machine is to be entirely made of stainless steel.

- Control panel must include Arabic language with clear display of all parameters   and functions.

- Printed documentation for complete treatment cycle, indicating success of process

- Ability to conduct biological indicator test to ensure reliability of the system.

- Error notification.

- No consumables needed such as special loading bags or disinfectants. System shall only require power, water and drainage and/or local steam supply.

- Water treatment such as softener must be included.

- Shredding System:

- Heavy duty; Indicating metal configuration and expected life before replacement of shredder certified from manufacture.

- Able to change direction at regular intervals to overcome jamming.

- Able to fragment surgical stainless-steel items. Please specify limitations if any!

- Proper feeding mechanism that guarantees all waste is fed into the shredder.

- Easy access for maintenance.

- Automatic detection of jamming with alarm notification and auto-stop mechanism as well as Emergency-Stop button.

- Bidders are requested to offer available loading solutions i.e. manual fully automated, semi-automated with a quoted price.

- Shredder must be integrated with the rest of the machine and not mounted to the machine to ensure synchronization of the process.

- Shredding must be prior to sterilization.

- Shredder must be steam sterilized after/through each cycle.

- In case of shredding interruption (jamming) safety measures should ensure that the load will be safe-to-handle by the maintenance staff.

- Sterilization System:

- Steam produced by a dedicated steam generator.

- Ability to work on domestic steam source as well.

- Cycle parameters (time, temperature, and pressure) are to be stated clearly.

- System must guarantee a high Sterility Assurance Level (SAL) according to international standards. SAL must be stated clearly.

- Integrated cooling system that guarantees a safe handling temperature for treated residue.

- Steam must be directed in contact with the load.

- Printed cycle parameters indicating success of sterilization process according to temperature, pressure and time with error indicator to repeat cycle.

- All security and safety measures such as prevention of door opening while system is hot must be listed clearly.

- Bidders are to state cycle requirements without ambiguity (Steam, water by type, electric power, compressed air).

- All optional accessories are to be listed and priced separately.

- Service Contract:

- Offers must include the annual price of a comprehensive services contract including replacement parts and labor valid for a period of five years starting at the end of the warranty.

- The total cost of ownership of the equipment over a period of seven years shall form the basis for a financial comparison between competing bidders.

SPECIAL TERMS

- Offers not complying with any of the special terms or the technical specifications shall be considered non-conforming with tender requirements.

- Any vendor providing FORGED documents shall be disqualified from the current tender and banned from participating in any future RMS tenders.

- All equipment must be the most recently released model/version which is equal to or higher than the range of the specifications of the required system (low, mid or high) and equal to or higher than the level of technology and required options mentioned in the technical specifications.

- Required certificates:

- For equipment of US origin, a copy of a certificate of FDA approval & the relevant 510K clearance for selling to US healthcare facilities for the offered model must be submitted with the technical offer.

- For equipment of other origins, a copy of either a CE certificate with the relevant CE number (MDD)/TÜV/BSI/UL OR a certificate of FDA approval & the relevant 510K clearance for selling to US healthcare facilities for the offered model must be submitted with the technical offer.

- Only for class Ι medical equipment, submission of a copy of Declaration of Conformity certificate (MDD) for the offered model shall be accepted.

- In all of the above cases certificates must be formally endorsed by JFDA.

- Country of origin:

  - The country of origin of the main part(s) of the system must be one of the following:

USA, Canada, Japan, UK, Sweden, Finland, Denmark, Switzerland, Belgium, Germany, France, Netherlands, Spain, Norway, Italy, Ireland, Austria, New Zealand, or Australia.

  - Accessories and consumables (as determined by the purchasing committee) may be manufactured in other countries and/or by different manufacturers.

  - All offered items must be approved for sale in the same country of origin.

  - Vendors must specify the origin of all offered items and accessories in the technical offer.

  - Equipment manufactured by reputable companies based in any of the countries mentioned in (3.1) will be taken into consideration regardless of the manufacturing site only:

  - If they are approved for sale in the same county of origin (an original and officially endorsed free-sale certificate from an authorised body must be included in the offer.

OR

  - If they are approved for sale in at least three of the countries mentioned in (3.1) (an original and officially endorsed free-sale certificate from an authorised body in those countries must be included in the offer).

  - Bidders must submit their reservations/queries regarding tender specifications and/or special terms within the first third of the tender closing period starting from the tender announcement date. Reservations/queries submitted after the end of this period shall be rejected.

- Warranty:

  - Offers must include a full warranty including spare parts and labour for a minimum period of 24 months from the date of installation and commissioning

  - If at any time during the warranty period the item becomes inoperative due to a technical fault the item must then be repaired by the supplier /local agent within a period of fourteen days from written notification, otherwise the supplier must replace the item with a new identical functioning one and will endure a penalty determined by the Royal Medical Services for each day of the downtime of the system. In case the item was replaced by a new one, the warranty period mentioned in (4.1.) above will start from the installation and commissioning date of the new item.

- One set of operation manual(s) and one set of service manual(s) including schematics and a spare-part list must be delivered with each unit, CD/DVD is acceptable. For large tenders, a certain agreed percentage of manuals per item may be agreed upon.

- Where applicable, pre-installation shall be the sole responsibility of the supplier. Pre-installation shall include removal of old system(s), any civil work, electrical work or site modification(s) necessary to accommodate the new system(s) according to manufacturers’ specifications and safety standards in addition to the work required for bringing back the site to the same working conditions as before installing the new system(s).

- Power requirements: where applicable either single phase 220V, 50Hz or 3-phase 380V. Systems with external transformers are considered conforming only if clearly stated in the technical specifications.

- Technical offers must include clear original technical brochures/catalogues for all offered items.

- Offers must include fully detailed technical offers and compliance sheets as a soft copy (either Microsoft office or Microsoft excel format) in addition to a hard copy, mentioning the exact model/catalogue number and country of origin of the offered item(s), full technical description/specifications and any accessories or options included in the offer.

- Compliance sheets must be as per the tabular format of the technical specifications in the tender documents, listing the required specifications on one column and a  Yes  or NO  response to each point in the adjacent column, with reference to page and line numbers in the relevant technical brochure. Offers not complying with this term shall be rejected.

- Accessories and consumables:

  - Any accessories and consumable items necessary to operate the offered system must be clearly identified and priced separately.

  - Technical offers must include a priced list for accessories and consumables as a hard copy in addition to a soft copy (either Microsoft office or Microsoft excel format) with prices fixed for a period of five years from the date of installation and commissioning with a maximum annual increase of 2%, any essential item not listed will be considered free of charge.

  - Accessories and consumables must be priced according to their delivery destination either to Queen Alia International Airport or to RMS Main Medical Stores.

  - Where applicable, a start-up kit of accessories and consumable items must be provided with each system on a free-of-charge basis.

- Spare Parts:

  - Technical offers must include a comprehensive and priced spare parts list as a hard copy in addition to a soft copy (either Microsoft office or Microsoft excel format) valid for a minimum period of five years with a maximum annual increase of 2%, commencing at the end date of the warranty period, any essential item not listed will be considered free of charge.

  - Spare parts must be priced according to their delivery destination either to Queen Alia International Airport or to RMS Main Medical Stores.

- Spare parts, consumables and accessories availability must be guaranteed for a minimum period of ten years starting from the date of installation and commissioning.

- Tender Awards:

  - For the final list of offers having a chance of winning the award, the awarding process shall be based on the accumulative value of both the offered item and its’ running cost (Total Cost of Ownership) over a period of seven years from the date of installation and commissioning. Only offers with the lowest total cost of ownership over a period of seven years from the date of installation and commissioning shall qualify for the award.

  - Running cost includes the value of consumables, accessories needed to operate the system over the same period as well as the cost of any service contracts (where applicable).

- For PC/Laptop based systems:

  - Complete restoration medium (CD/DVD/etc.) of the operating system and the application software must be supplied.

  - Where locally supplied computers or laptops are offered, only computers/laptops from Apple, hp/Compaq, Lenovo, Dell, fujtisu or Toshiba will be accepted, offered models must be the latest available version upon delivery.

  - Where locally supplied printers are offered only the following types and brands are accepted: HP, SAMSUNG, OKI, CANON, EPSON.

- Pricing must include services of sale, shipment, transportation, delivery from port to site or to Main Medical Stores, installation, pre-installation (if needed), training, commissioning, warranty and bringing the equipment into service.

- Custom clearance of goods shall be the responsibility of the Jordanian Armed Forces (JAF), however, suppliers shall bear all costs incurred by handling charges and any demurrage charges or extra expenses incurred by the port’s corporation (including expenses caused by delay in presenting the necessary shipment documents for either clearing or transporting the goods to the required location mentioned in the final order, delivery note issuing charges, unloading charges, local shipping charges etc.). The supplier is also responsible for providing of all relevant shipping documents, together with the delivery order(s).

- DRMS has the right to increase or decrease the awarded quantities by a percentage not exceeding 30% after the final order notification with the same prices, terms and conditions of the original contract upon DRMS request and approval of the awarded party.

- The supplier must furnish DRMS with a guarantee stamped and legalized by the Notary Public equals to (115%) of the total value of the awarded equipment valid for twelvemonths from the date of final acceptance of the equipment by DRMS.

- Training:

  - For items where service training courses for the offered system are usually conducted abroad, offers must include a certified service training program at a reputable centre abroad recognized by the manufacturer for one RMS engineer; all costs inclusive, air tickets, boarding, commuting, accommodation and any extra costs.

  - The period of the training courses must be in accordance with the manufacturer’s program, with a minimum duration of at least five working days excluding travelling days, and must be stated clearly in the technical offer.

  - Training Programs must conform to the following standards:

    - Operator training must comprise understanding and use of operation manual(s), correct and safe operation of the equipment, as well as user preventive maintenance and calibration.

    - Service training must comprise: theory, understanding and use of service manual(s), calibration, preventive maintenance procedure, and practical troubleshooting and repair exercises, and must be conducted by professional instructors employed or authorized by the system manufacturer.

    - Service training must be conducted on a system of identical make, model, and configuration to that purchased by DRMS, and designated by the manufacturer or the local agent for training purposes.

    - Certificates must be endorsed and officially sealed by the system manufacturer, legally empowering trainees to engage in user and service activities according to operation and service manual(s).

    - Where applicable, offers must include on-site user and service training.